/*
 * Cycle assist in the Sanyo/Sachs hub. 
 * Read out sensor values and determine
 * its support from that. January 2020
v5 update: read battery temperature during acceleration. If low, decrease PWM instead of sudden power limit.
*/


const int Thermistor_pin_out  = 4; // 5V for thermistor readout
const int Thermistor_pin_in   = A1; // Analog thermistor voltage readout
const int Batt_Volt_pin_in    = A2; // Analog battery voltage readout
const int driver_pin          = 3; // Pin that steers the motor driver.
bool ifdo_lim25               = true; // global boolean, whether 25 km/h speed limit should be enforced.
int Drive_PWM                 = 0; // define the initial drive support variable
int Drive_PWM_recommend       = 0; // the recommended drive support from PAS and speed sensor.
int Drive_PWM_max_safe        = 0; // The maximum drive support according to safety checks.
bool ifdo_PWR_red_temp        = true; // initialize the 'safety check' booleans
bool ifdo_PWR_red_batt        = true;
bool ifdo_disable_batt        = true;
const int Precharge_driver_pin = 2; // Digital 2 is the MOSFET driver input pin.

// REED SENSOR/WHEEL SPEED:
const int Reed_pin            = 5; // Pin which reads the reed (speed) sensor.
unsigned long reed_2_LOW      = 0; // time stamp of the last jump from LOW to HIGH of reed readout.
unsigned long reed_2_HIGH     = 0; // time stamp of the last jump from HIGH to LOW of reed readout.
double wheel_speed            = 0; // Current speed [km/h] of the wheel.
double Batt_volt              = 0; // Battery Voltage [V]

// HALL SENSOR/WHEEL SPEED:
const int PAS_pin             = 6; // Pin which reads the PAS (pedal) sensor.
unsigned long PAS_2_LOW      = 0; // time stamp of the last jump from LOW to HIGH of Hall sensor readout.
unsigned long PAS_2_HIGH     = 0; // time stamp of the last jump from HIGH to LOW of Hall sensor readout.
double cadence                = 0; // Current speed [Hz] of the pedal rotation.

//TCCR2B = TCCR2B & B11111000 | B00000010; // set timer 2 divisor to 8 for PWM frequency of 3921.16 Hz (channel D3 & D11)

void setup() 
{
  // Upon startup:
  Serial.begin(115200);
  // Read if run in 25km-mode or 25km+mode:
  check_25km_limit(PAS_pin);
  TCCR2B = TCCR2B & B11111000 | B00000010; // set timer 2 divisor to 8 for PWM frequency of 3921.16 Hz (channel D3 & D11)
  
  // Initial settings:
  digitalWrite(Thermistor_pin_out, HIGH); // Set thermistor readout output pins to high
  digitalWrite(Precharge_driver_pin, LOW); // Capacitor needs to charge up first.
  // set the REED sensor to HIGH as default:
  pinMode(Reed_pin, INPUT_PULLUP);
  digitalWrite(driver_pin, HIGH); // No drive support at startup.
  delay(2000); // 3000 seems OK for the capacitor to fill up
  digitalWrite(Precharge_driver_pin, HIGH);
  pinMode(Precharge_driver_pin, INPUT_PULLUP);

}

void loop() {
  // First perform safety checks, and determine the maximum Drive support the motor can safely provide:
  Drive_PWM_max_safe = Determine_PWM_max_safe(Thermistor_pin_in, Batt_Volt_pin_in);

  // Read the current state of the wheel speed sensors, and if possible, update the wheel speed:
  update_wheel_speed(Reed_pin);
  // Read the current state of the pedal speed sensors, and if possible, update the pedal speed:
  update_cadence(PAS_pin);
  // With that information, update the recommended drive support level:
  update_drive_support(Drive_PWM_max_safe);
  // Write the signal to the motor driver:
  analogWrite(driver_pin, Drive_PWM);
  delay(9);
  print_drive_variables();
  print_safety_variables();
}

void check_25km_limit(int PAS_pin)
{ // We wait a while to see if the pedals are moved sufficiently in those first seconds:
  unsigned long t_0 = millis(); // Time the start
  unsigned long max_wait_total = 1000; //[msec]
  while ((millis() - t_0) < max_wait_total)
  { // Measure the cadence:
    update_cadence(PAS_pin);
  }
  Serial.print("Startup cadence: \n"); Serial.print(cadence);  Serial.print("\n");
  if (cadence > 50)
  { // If the cadence has risen enough from zero, we lift the 25 km/h limitation:
    ifdo_lim25 = false;
  }
  cadence = 0; // reset the cadence to zero.
  Serial.print("25 km/h limit:\n");
  Serial.print(ifdo_lim25);
  Serial.print("\n");
}

// Local function: print variables for debug:
void print_safety_variables(void)
{ 
  Serial.print("\n 25 lim:");
  Serial.print(ifdo_lim25);
  Serial.print("\t red_temp?:");
  Serial.print(ifdo_PWR_red_temp);
  Serial.print("\t Voltage?:");
  Serial.print(Read_battery_voltage(Batt_Volt_pin_in));
  Serial.print("\t Low voltage?:");
  Serial.print(ifdo_PWR_red_batt);
  Serial.print("\t Empty battery?:");
  Serial.print(ifdo_disable_batt);
}
void print_drive_variables(void)
{ 
  Serial.print("\n wheel_speed:");
  Serial.print(wheel_speed);
  Serial.print("\t cadence:");
  Serial.print(cadence);
  Serial.print("\t PWM_max_safe:");
  Serial.print(Drive_PWM_max_safe);
  Serial.print("\t Drive_PWM:");
  Serial.print(Drive_PWM);
} 

// Local function, determine the maximum Drive support the motor can safely provide:
int Determine_PWM_max_safe(int Thermistor_pin_in, int Batt_Volt_pin_in)
{
  // Check whether motor temperature is too high for full power:
  ifdo_PWR_red_temp = Read_motor_temp(Thermistor_pin_in);
  // Then check the battery voltage, if it is low, we should reduce power:
  ifdo_PWR_red_batt = is_Batt_low(Batt_Volt_pin_in);
  // HACK: we now do this at the PWM determination:
  ifdo_PWR_red_batt =  false;
  // Then check the battery voltage, if it is empty, we should disable the drive:
  ifdo_disable_batt = is_Batt_empty(Batt_Volt_pin_in);
  // From the above safety checks, determine the maximum drive PWM duty cycle possible:
  int Drive_PWM_max = drive_support_max_safe(ifdo_PWR_red_temp, ifdo_PWR_red_batt, ifdo_disable_batt);
  return Drive_PWM_max;
}

// Local function: read out the motor temperature, decide whether reduced power mode should be enabled:
bool Read_motor_temp(int Thermistor_pin_in)
{
  // The thermal resistor value is read out by a voltage divider bridge 
  // consisting of the thermistor and an external resistor.
  int ext_resistor = 12; // [kOhm]
  // At critically high temperatures, the thermistor has a value of:
  int thermistor_threshold = 8; // [kOhm]
  // Now measure the voltage and compare to the critical one:
  int readout_threshold = 800 ;//750 seems reasonable. 1023*ext_resistor/(ext_resistor + thermistor_threshold);
  double readout = 0.00;
  //while (readout == 0.00)
  readout = analogRead(Thermistor_pin_in);
  int PWR_red = true;
  if (readout < readout_threshold)
  {
    PWR_red = false;
  }
  Serial.print("\t Thermistor reading:");
  Serial.print(readout);
  return PWR_red;
}

// Local function: judge whether the drive needs to reduce power, or disable, due to too low battery voltage:
bool is_Batt_low(int Batt_Volt_pin_in)
{
  bool PWR_red = true;
  // Define the constants and thresholds:
  int thres_PWR_red_volt = 3.2*13; //[Volt] Below this voltage, reduced power mode should be enabled.
  // Read the battery voltage:
  double Batt_volt = Read_battery_voltage(Batt_Volt_pin_in);
  
  // Compare the battery voltage to the thresholds for reduced power and drive disable:
  if (Batt_volt > thres_PWR_red_volt)
  {
    PWR_red = false;
  }
  return PWR_red;
} 

// Local function: judge whether the drive needs to disable, due to too low battery voltage:
bool is_Batt_empty(int Batt_Volt_pin_in)
{
  bool ifdo_drive_disable = true;
  // Define the constants and thresholds:
  int thres_PWR_drive_disable = 3.1*13; //[Volt] Below this voltage, the drive should be disabled.
  // Read the battery voltage:
  double Batt_volt = Read_battery_voltage(Batt_Volt_pin_in);
  // Compare the battery voltage to the thresholds for reduced power and drive disable:
  if (Batt_volt > thres_PWR_drive_disable)
  {
    ifdo_drive_disable = false;
  }
  return ifdo_drive_disable;
} 

// Local function: read out battery voltage, to be within safe cell voltage limits:
double Read_battery_voltage(int Batt_Volt_pin_in)
{
  // The battery voltage is read by constructing a voltage dividing bridge, with the resistor values:
  float R1  = 24; //[kOhm] Resistor from Battery plus to analogRead pin.
  float R2  = 2.4; // [kOhm] Resistor from analogRead to GND.
  float Batt_volt_readout = analogRead(Batt_Volt_pin_in);
  

  double current_Batt_volt = (double) (R1+R2)/(R2) * Batt_volt_readout*5/1024;

  Batt_volt = (20*Batt_volt + current_Batt_volt)/(20+1); // [V] 

  return Batt_volt;
}

// Local function to define a maximum Drive support, using the safety checks performed before:
int drive_support_max_safe(bool ifdo_PWR_red_temp, bool ifdo_PWR_red_batt, bool ifdo_disable_batt)
{
  // Determine the maximum amount of support we can safely deliver:
  int PWM_max = 180;
  // Check whether the power should be reduced:
  if (ifdo_PWR_red_temp || ifdo_PWR_red_batt)
  { PWM_max = 80;}
  if (ifdo_disable_batt)
  { PWM_max = 0;}
  return PWM_max;
}

// Local function to calculate the current wheel rotation speed, if possible:
void update_wheel_speed(int REED_pin)
{
  // 25 km/h: 1/12.5= 80 ms between reed pulses
  double dt_HIGH_max = 500; //[msec] Larger than this means wheel standstill
  double dt_LOW_max = 400; //[msec] Larger than this means wheel standstill
  double nof_running_averages = 20; // The cadence is updated with running averages.

  unsigned long t_now = millis();   // [msec] measure the current time
  int reed_status = digitalRead(REED_pin); //'LOW' (means that a magnet is in front of the sensor, high means there is not):
 
  // Which status was read last?
  if (reed_2_HIGH == 0 && reed_2_LOW == 0)
  { // This probably means that neither value has ever been recorded, we therefore have no speed information and recommend no support:
    wheel_speed = 0;
    if (reed_status == LOW)
    { reed_2_LOW = t_now;  } // Initialize the first timestamp}
    if (reed_status

== HIGH)
    { reed_2_HIGH = t_now;  }// Initialize the first timestamp}
    return;
  }
  
  if (reed_2_HIGH > reed_2_LOW){ // The switch from LOW to HIGH was read last:
      //Serial.print("\t LOW to HIGH switch was read last");
    if (reed_status == LOW) { // This means we have a switch of level compared to the last readout. 
      // We overwrite the reed_2_LOW time stamp:
      reed_2_LOW = t_now;
    }
    // If the level has not switched, it might have been too long in the same status (standing still):
    if ((reed_status == HIGH && (t_now - reed_2_HIGH) > dt_HIGH_max) || reed_2_LOW == 0)
    { // This means the pedals are practically standing still, or have not moved yet:
        wheel_speed = 0;//recommend no support.
        //Serial.print("\t Too long in HIGH! no support");
        return;
    }
    return;
  }

  if (reed_2_HIGH < reed_2_LOW)  { // The 'LOW' status was read last:
  //Serial.print("\t HIGH to LOW switch was read last");
    if (reed_status == HIGH) { // This means we have a switch of level. 
      // We overwrite the reed_2_HIGH time stamp:
      reed_2_HIGH = t_now;
      // We can measure how long this switch of level took:
      double dt_HIGH = (t_now - reed_2_LOW);
      // From this, we can measure the speed:
      double current_wheel_speed = (25*80/2)/dt_HIGH;
      // Here we assume that the wheel speed is within acceptable limits:
      if (current_wheel_speed > wheel_speed){ // If the cadence increases, we let it increase slowly through running average:
        wheel_speed = (nof_running_averages*wheel_speed + current_wheel_speed)/(nof_running_averages+1); // [km/h] 
      }
      if (current_wheel_speed <= wheel_speed){ // If it has decreased, we must act quickly in case of a stop.
        wheel_speed = current_wheel_speed; // [km/h]
      }
    }
    if ((reed_status == LOW && (t_now - reed_2_LOW) > dt_LOW_max) || reed_2_HIGH == 0)
    { // This means the pedals are practically standing still:
       wheel_speed = 0;//recommend no support.
       //Serial.print("\t Too long in LOW! no support");
        return;
    }
    return;
  }
}

// Local function to calculate the current wheel rotation speed, if possible:
void update_cadence(int PAS_pin)
{
//  // 8 magnets in the PAS sensor, 8 block signals per rotation
//  // The minimum rotation speed of the pedal is 0.5 Hz, so 4 Hz (250 millisecond) for the signal:
  double dt_HIGH_max = 400; //[msec] Larger than this means standstill of pedals
  double dt_LOW_max = 250; //[msec] Larger than this means standstill of pedals
  double nof_running_averages = 3; // The cadence is updated with running averages.

  unsigned long t_now = millis();   // [msec] measure the current time
  int PAS_status = digitalRead(PAS_pin); //'LOW' (means that a magnet is in front of the sensor, high means there is not):
 
  // Which status was read last?
  if (PAS_2_HIGH == 0 && PAS_2_LOW == 0)
  { // This probably means that neither value has ever been recorded, we therefore have no speed information and recommend no support:
    cadence = 0;
    if (PAS_status == LOW)
    { PAS_2_LOW = t_now;}
    if (PAS_status == HIGH)
    { PAS_2_HIGH = t_now; }
  }
  
  if (PAS_2_HIGH > PAS_2_LOW){ // The switch from LOW to HIGH was read last:
      //Serial.print("\t LOW to HIGH switch was read last");
    if (PAS_status == LOW) { // This means we have a switch of level compared to the last readout. 
      // We overwrite the PAS_2_LOW time stamp:
      PAS_2_LOW = t_now;
    }
    // If the level has not switched, it might have been too long in the same status (standing still):
    if ((PAS_status == HIGH && (t_now - PAS_2_HIGH) > dt_HIGH_max) || PAS_2_LOW == 0)
    { // This means the pedals are practically standing still, or have not moved yet:
        cadence = 0;//recommend no support.
        //Serial.print("\t Too long in HIGH! no support");
        return;
    }
    return;
  }

  if (PAS_2_HIGH < PAS_2_LOW)  { // The 'LOW' status was read last:
  //Serial.print("\t HIGH to LOW switch was read last");
    if (PAS_status == HIGH) { // This means we have a switch of level. 
      // We overwrite the PAS_2_HIGH time stamp:
      PAS_2_HIGH = t_now;
      // We can measure how long this switch of level took:
      double dt_HIGH = (t_now - PAS_2_LOW);
      // From this, we can measure the speed:
      double current_cadence = max(max(dt_HIGH_max, dt_LOW_max) - dt_HIGH, 1); // [Hz]
      //Serial.print("\t current cadence: "); Serial.print(current_cadence);
      // Here we assume that the cadence is within acceptable limits:
      if (current_cadence > cadence){ // If the cadence increases, we let it increase slowly through running average:
        cadence = (nof_running_averages*cadence + current_cadence)/(nof_running_averages+1); // [Hz] 
      }
      if (current_cadence <= cadence){ // If it has decreased, we must act quickly in case of a stop.
        cadence = current_cadence; // [Hz]
      }

    }
    if ((PAS_status == LOW && (t_now - PAS_2_LOW) > dt_LOW_max) || PAS_2_HIGH == 0)
    { // This means the pedals are practically standing still:
       cadence = 0;//recommend no support.
       //Serial.print("\t Too long in LOW! no support");
        return;
    }
    return;
  }
}

// Local function to determine the Drive_PWM:
int update_drive_support(int PWM_max_safe)
{
  // Set constants and Parameters:
  int dPWM_max_increase   = 1; // Maximum increase of drive support compared to the previous
  int dPWM_max_decrease_25= 2; // Maximum decrease of drive support compared to the previous, in 25 km/h mode.
  float cadence_minimum = 80; // Minimum cadence, below no drive support
  float wheel_speed_minimum = 0.3; // Minimum wheel speed, below that: no drive support
  // 25 km/h: 1/12.5= 80 ms between reed pulses
  double dt_25  = 80; //[msec]

  // Determine whether the 25 limit is enabled:
  if (ifdo_lim25 == 1 && wheel_speed > 25) {// We are driving faster than 25 km/h, while we should not, reduce support:
    Drive_PWM = max((Drive_PWM - dPWM_max_decrease_25), 0); 
    Serial.print("\n Limiting PWM \n");
  }
  // Disable the support if the cadence or speed is at zero, or safety checks command a stop of support:
  if (wheel_speed <= wheel_speed_minimum || cadence <= cadence_minimum || PWM_max_safe == 0) { 
    Drive_PWM = 0;
    return 0;
  }
  else if (is_Batt_low(Batt_Volt_pin_in)) // This means the battery voltage is currently low, so we limit the PWM a bit:
    { 
      Drive_PWM = Drive_PWM - dPWM_max_decrease_25;
    }
  else 
    { // Write the PWM_drive, with the limitation of the maximum increase of drive support:
      Drive_PWM_recommend = (cadence-cadence_minimum)*(cadence-cadence_minimum)*5/1000 + (wheel_speed-wheel_speed_minimum)*0.0;
      Drive_PWM = min(Drive_PWM + dPWM_max_increase, Drive_PWM_recommend) ;
    }
  // Overwrite the drive PWM in case of any Power reduction or disabling:
  Drive_PWM = min(Drive_PWM, PWM_max_safe);
  // Limit the speed when driving over 30 km/h:
  int PWM_limit_high_speed = 165;
  int wheel_speed_limit_start = 30;
  if (wheel_speed > wheel_speed_limit_start)
  {
    PWM_limit_high_speed = PWM_limit_high_speed - (wheel_speed - wheel_speed_limit_start)*3;
  }
  
  Drive_PWM = min(Drive_PWM, PWM_limit_high_speed); // Limit the Drive_PWM such that the thing doesn't start flying.
  return Drive_PWM;
}